R.L.E

The R.L.E. is a 1 1/4" Bore 2" Stroke Hit & Miss Governed Water Hopper Cooled O/C Engine.  It will run on both Gas or Petrol.  The Engine was designed with simplicity of machining in mind and requires a lathe of around 3 1/2" swing to machine the flywheels, all other castings can be managed quite easily.  The engine runs on cast iron main bearings for simplicity, but brasses can be fitted if desired.

CASTINGS PROVIDED

Engine base.			Base carb.

Main frame.			Rocker support.

Cylinder.			Piston (without rings)

Cylinder head.			Two 6 3/4 Diameter flywheels.

Main bearing caps.		Silencer (Amanco style)

Water hopper.			Brass plate.

All in machine grade cast iron.

Un machined weight... 25lbs		Machined weight... 23lbs

Overall dimentions... L 14".  W 8".  H 8"

Full engineering drawings

Extra Parts

These parts are made by the constructor.

2" Throw crankshaft... Con rod fabricated from 1 by 1/4 B.D.M.S....  Big end bearing from 1" dia. P.B....  Rocker and governor latch from 1/4 by 1/4 B.D.M.S. 2:1 Gears on 975 thou C.D....  Push rod from 3/16 dia. S.S.... Nuts...  Bolts...  Screws...  Oiler... etc.

This set of castings C/W drawings will provide, once finished, a Spake Ignition Twin F/Wheel, Hit & Miss Petrol Engine.