A.E. & H. ROBINSON'S

"X" TYPE GAS ENGINE

HALF SCALE

This engine was first introduced in 1890 using a star wheel for operating the exhaust valve. In 1893/4 Horace Robinson patented the epicyclic drive which was fitted to this model. Known as the oblique design it was found to be a most reliable and compact unit and was used to power the Robinsons range of catering equipment. The machines became affectionately known as "CHIPPIE" engines.

Our engine kit is a true 1/2 scale replica and differs only in the way the cylinder water jacket is provided from the original. Its bore and stroke are 1 1/2" by 1 1/2" or 38.1mm by 38.1mm.

CASTINGS PROVIDED		TOTAL WEIGHT 30.8lbs 14kgs

Water base			Flywheel @ 8 3/4" dia.

Main bearing caps		Eccentric strap.

Cylinder				Hot tube chimney and cap.

Cylinder liner			Inlet valve chest.

Epicyclic gearbox casing		Exhaust valve chest.

Piston				Brass No. plate.

All castings in finest machine grade iron.

Overall dimentions, H 9".   W 7".   L 11".

Pair of gears provided, External in E.N.32, special internal diecast in ZA8.

Parts found by the constructor.

1 1/2" throw crankshaft... connecting rod, from 1 1/4 by 3/8 by 5" B.D.M.S. Ignition tube from 3/8 dia. by 2 1/2" Silver Steel... Valves from 3/8" dia. by 2" S.S. & 3/4" dia. by 1 1/2 S.S.... Tube clamp from 2" by 1/2" by 1/8" B.D.M.S. Chimney support from 1/4" dia. by 2" B.D.M.S.... 1/8 GibHead Key... Nuts... Bolts... Oiler... etc.

This set of castings C/W drawings will provide, once finished, a Tube Ignition Single F/Wheel Gas Engine.